Smart Trolley with Controlled Movement and QR-Based Billing

Introduction

This project aims to enhance the shopping experience by integrating an RFID-based smart trolley system with automated billing and QR code-based payment. The system consists of two Arduino boards: one for RFID-based item scanning and another for displaying the total bill on an OLED screen while generating a dynamic UPI QR code for payment.

Components Used

- Arduino Uno (x2) – One for RFID scanning and another for OLED display.

- RFID Module (MFRC522) – To scan tagged items.

- I2C LCD Display (16x2) – To display item details and total bill.

- OLED Display (0.96-inch, 128x64 SSD1306) – To generate and display the QR code.

- IR Sensor – To detect item removal.

- Buzzer & LED – To provide audio-visual feedback.

- Push Button – To enable item removal.

- Wires and Connectors – To connect all components.

- Power Supply – Battery pack for Arduino.

Working Principle

1. Item Scanning and Billing

- Each product has an RFID tag with a unique ID.

- When an item is scanned, the Arduino reads the tag and compares it with a predefined list.

- If the tag matches an item in the database, its name and price are displayed on the LCD.

- The total bill is updated, and a buzzer/LED provides confirmation.

- If the payment RFID card is scanned, the final bill is transferred to the OLED display for QR code generation.

2. Item Removal Process

- Pressing the push button enables item removal mode.

- The user scans the RFID tag of an item to remove it.

- If the scanned tag matches an added item, it is removed from the bill.

- The total is updated, and feedback is provided through the LCD and buzzer.

3. QR Code-Based Payment

- The second Arduino receives the total bill amount serially from the first Arduino.

- It generates a dynamic UPI QR code using the format:

- upi://pay?pa=8074468294@ybl&pn=Payment&am=<amount>&cu=INR&tr=<txnRef>

- The QR code is displayed on the OLED screen.

- The customer scans the QR code using a UPI-enabled app (e.g., PhonePay, Google Pay) to complete the transaction.

System Flow

1.Start-Up

- Displays a welcome message on the LCD and OLED.

2.Item Scanning

- Reads RFID tag and updates the total.

- Displays the item name and price.

3.Item Removal (if needed)

- Activated by pressing the button.

- Removes the scanned item and updates the total.

4.Payment Process

- When the payment RFID tag is scanned, the total bill is displayed on the OLED.

- The QR code is generated and displayed.

- The user completes the payment via UPI scanning.

4.Completion

- Displays a thank-you message.

- Resets the total amount for the next transaction.

Objectives of the Project

- To develop an automated billing system for supermarkets using RFID technology.

- To minimize waiting time at checkout counters.

- To integrate a cashless payment system using dynamic QR codes.

- To provide real-time feedback through LCD and OLED displays.

- To enhance user experience with a smart, interactive trolley system.

Software Used

Software & Libraries

- Arduino IDE for coding and uploading sketches.

- Libraries Used:

  - <Wire.h> – Handles I2C communication.

  - <LiquidCrystal_I2C.h> – Controls the I2C LCD.

  - <SPI.h> – Enables SPI communication for the RFID module.

  - <MFRC522.h> – Reads RFID tags.

  - <Adafruit_GFX.h> & <Adafruit_SSD1306.h> – Controls the OLED display.

  - <qrcode.h> – Generates QR codes dynamically.

Implementation Details

- RFID Integration: Each product has an RFID tag. When scanned, its details are matched with stored data, and the price is updated on the LCD.

- Serial Communication: Two Arduino boards communicate via serial connection to send the total bill for QR code generation.

- Item Removal: If an item needs to be removed, pressing the push button enables removal mode, and the item is deducted from the total.

- Payment Processing: The OLED screen generates a dynamic UPI QR code, which users can scan using payment apps like Google Pay or PhonePe.

### Advantages of the Smart Trolley

- Reduces human effort in scanning and billing.

- Avoids long queues at billing counters.

- Provides a seamless shopping experience with automated checkout.

- Encourages digital payments and reduces cash handling.

- Easy-to-use interface with instant feedback through LED and buzzer alerts.

## Conclusion

The Smart Trolley System revolutionizes the shopping experience by integrating RFID-based item scanning and QR code-based billing. By reducing manual efforts, streamlining checkout, and encouraging digital payments, this system significantly enhances efficiency in supermarkets and retail stores. Future enhancements, such as Wi-Fi integration and AI-powered analytics, can further improve the system, making it a valuable innovation in the retail industry.